Saliva-Based Wearable Biosensor for Continuous Cardiac Biomarker Monitoring

Research Landscape, Commercialization Pain Points, and Proposed Mouthguard Biosensor Solution

# 1. Purpose of This Document

This document explains why continuous monitoring is especially critical for heart failure, summarizes recent research and near-commercial efforts detecting cardiac biomarkers (troponin, BNP/NT-proBNP) without a blood draw, identifies the specific reasons a saliva-based cardiac biosensor has not yet been commercialized, and lays out how our proposed mouthguard-based saliva biosensor is designed to close that gap.

# 2. Why Continuous Monitoring Matters for Heart Failure

## 2.1 The post-discharge danger window

Heart failure hospitalizations remain a massive and recurring burden: in the United States alone, more than 1 million people are hospitalized for a primary diagnosis of heart failure annually. The period immediately after discharge is especially dangerous — clinicians describe a distinct "vulnerability phase" during which patients are at their highest risk of decompensating again, and studies have shown that after a first heart-failure admission, mortality can reach up to 50% within one year. Yet this is exactly the period when patients have the least clinical oversight, seen only at scheduled follow-up visits rather than continuously.

## 2.2 Proof that continuous monitoring changes outcomes

This is not a theoretical benefit — it has already been demonstrated with an existing (invasive) continuous monitoring device, CardioMEMS, an implantable pulmonary-artery-pressure sensor. In the pivotal CHAMPION trial, patients managed with continuous pressure monitoring saw a 58% reduction in all-cause hospital readmissions and a 78% reduction in heart-failure-specific readmissions, and a separate analysis found heart failure hospitalizations dropped from 1.24 per patient in the year before implantation to 0.52 per patient in the year after. The mechanism is straightforward: continuous monitoring lets clinicians detect fluid build-up and rising cardiac pressure days before visible symptoms such as breathlessness or swelling appear, allowing medication adjustments that prevent the crisis entirely rather than reacting to it after the fact.

CardioMEMS proves the underlying principle our proposal depends on: continuous, trend-based physiological data — not periodic snapshots — is what allows early intervention. Our mouthguard biosensor applies this same principle to a biochemical signal (cardiac biomarker trend) instead of a mechanical one (pressure), while removing the need for a surgical implant.

## 2.3 Why a biomarker trend specifically, not just a single reading

- Rate of change matters more than a single value: a rising troponin or NT-proBNP trend signals active, worsening cardiac stress, while a stable or falling trend at the same absolute level can reflect a controlled, non-urgent state — a distinction a single blood draw cannot make.

- Decompensation is frequently silent in its early stages: biomarker levels can begin rising before a patient consciously notices symptoms, so passive, continuous sensing does not depend on the patient recognizing something is wrong.

- Symptom-triggered testing is inherently reactive: today, a biomarker test only happens if a patient decides something feels wrong enough to seek care, by which point deterioration may already be advanced.

## 2.4 Two distinct patient populations this addresses

- Early / at-risk diagnosis: for people with risk factors for heart failure but no formal diagnosis yet, continuous biomarker trend tracking could flag an abnormal upward trend prompting earlier clinical evaluation, well before a first hospitalization ever occurs.

- Known heart-failure patients (post-discharge surveillance): for patients who have already been hospitalized for heart failure, continuous at-home monitoring fills the exact "vulnerability phase" gap described above — providing an ongoing early-warning signal between clinic visits so a second decompensation event can be caught and treated before it requires readmission.

# 3. Recent Work Detecting Cardiac Biomarkers from Non-Blood Fluids

No product exists today that detects troponin or BNP/NT-proBNP outside a blood sample. However, several research groups and one company have made real progress toward that goal, summarized below.

| Effort | Fluid / Route | Mechanism | Stage / Status |
| --- | --- | --- | --- |
| RCE Technologies — Infrasensor | Transdermal (through skin, optical) | Mid-infrared spectroscopy + ML to estimate troponin-I concentration at the wrist | Multicenter clinical trials; 90% accuracy predicting troponin-I/obstructed arteries in 5 min; not yet FDA-cleared or commercially sold |
| FIRST-SIGN (EU-funded consortium) | Interstitial fluid (microneedle) | Microneedle wearable benchmarking ISF troponin against venous blood | Active development project targeting CE marking; not yet a product |
| NIH-funded academic project | Interstitial fluid (microneedle) | Aptamer-based electrochemical sensing for continuous troponin-I | University research stage; not commercialized |
| Femtomolar NT-proBNP aptasensor (academic) | Saliva (artificial) | Liquid-gated field-effect transistor with reduced graphene oxide + NT-proBNP aptamer | Lab-validated in artificial saliva only; no company or in vivo continuous validation yet |
| Nitrogen-doped graphene oxide troponin electrode (academic) | Serum and saliva | Electrochemical detection of cardiac troponin I | Lab research stage |
| Photonic crystal biochip (academic) | Saliva and urine (one drop) | Printed immunosensing biochip with fluorescence enhancement; 10-minute troponin T detection | Validated against 100 clinical samples in lab study; not commercialized |
| Marathon-runner saliva/serum study (clinical) | Saliva | Point-of-care troponin-I immunoassay comparing saliva to serum across time points | Clinical feasibility study; confirmed correlated kinetic pattern between saliva and serum |

# 4. Why Saliva-Based Cardiac Biomarker Biosensors Have Not Been Commercialized

- Ultra-low biomarker concentration in saliva compared to blood, well below what most existing validated salivary sensors (glucose, uric acid, lactate) are designed to detect.

- No standardized, agreed-upon correlation between salivary and blood biomarker concentrations across studies.

- Saliva is a highly variable, easily contaminated sample matrix (flow rate, food/drink, talking).

- Biofouling in the oral cavity degrades sensor surfaces quickly; no cardiac-specific saliva sensor has demonstrated multi-day stability.

- No completed large-scale clinical validation or regulatory (FDA/CE) pathway for any saliva-based cardiac biomarker device.

- Practical wear compliance — a mouthguard competes with eating, drinking, and speaking, unlike a skin patch.

# 5. Our Proposed Solution: Mouthguard-Based Saliva Biosensor

A custom-fit intraoral mouthguard embedding an aptamer-functionalized, MXene-hydrogel electrode, designed specifically around the pain points above. Design layers:

| Layer | Component / Function |
| --- | --- |
| Form factor | Custom-fit, removable intraoral mouthguard (worn like a retainer) |
| Sensing electrode | Aptamer-functionalized electrode (troponin- and NT-proBNP-specific aptamers) on a MXene-hydrogel composite substrate |
| Pre-filtration layer | Porous hydrogel membrane controlling biomarker diffusion while screening out food debris and bacteria |
| Flow architecture | Filter/O-ring assembly with a via-hole design enabling continuous, uninterrupted saliva flow across the sensing surface |
| Antifouling layer | Cellulose-acetate-type coating to resist mucus, food particle, and bacterial buildup over extended wear |
| Signal transduction | Electrochemical aptamer-based (E-AB) / impedimetric sensing — reversible binding supports continuous, repeated measurement |
| Electronics & telemetry | Embedded low-power Bluetooth module transmitting readings to a companion phone app in real time |
| Data processing | Deep-learning-assisted analysis to separate genuine biomarker trend changes from noise caused by eating, drinking, or flow-rate variation |

# 6. How Our Proposal Fills the Gap

Each design choice above maps directly to a specific reason saliva-based cardiac biosensing has stalled in research and never reached the market:

| Pain point blocking commercialization | How our mouthguard biosensor addresses it |
| --- | --- |
| Ultra-low biomarker concentration: NT-proBNP in saliva can be ~1000x lower than in blood, down to pg/mL levels; troponin is already picomolar-range in blood before further dilution. | Aptamer-functionalized electrode built on a MXene-hydrogel composite. MXene's high electrical conductivity and sensitivity amplifies weak binding signals, the same principle behind lab aptasensors reaching femtomolar detection limits. |
| No standardized saliva-to-blood correlation yet — some studies report strong correlation, others only moderate correlation with high variability. | Built-in calibration/regression model trained against paired serum measurements during development, positioning the project as advancing correlation science rather than assuming it is already solved. |
| Messy, uncontrolled sample matrix — flow rate, food/drink contamination, and talking all introduce noise unrelated to the true biomarker signal. | A porous hydrogel membrane over the electrode acts as a biochemical pre-filtration layer, controlling biomarker diffusion while blocking food debris and non-specific material — the same principle validated in periodontal (MMP-8) mouthguard sensors. |
| Biofouling and short sensor lifespan — existing validated salivary mouthguard sensors have only demonstrated a few hours of stable continuous operation. | Antifouling coating (as used in validated salivary glucose mouthguards) combined with the hydrogel's inherent resistance to non-specific adsorption, targeting multi-day stability rather than hours. |
| No completed clinical validation or regulatory pathway — every saliva-cardiac-biomarker result to date is lab/academic-stage only. | Proposed as the next feasibility step toward pilot/clinical validation, at the same maturity stage as comparable interstitial-fluid efforts (FIRST-SIGN, NIH-funded work) — an honest, credible positioning rather than an overclaim. |
| Comfort and adherence for continuous wear — a mouthguard occupies the mouth and competes with eating, drinking, and speaking. | Realistic wear protocol: continuous during sleep/rest periods, removable during meals — mirroring how CGMs are actually worn in practice rather than claiming unrealistic 24/7 use. |

# 7. What Makes This Proposal Unique

No existing saliva-based cardiac biomarker work combines a MXene-hydrogel composite electrode with aptamer-based recognition; published academic efforts use graphene-oxide field-effect transistors or photonic-crystal biochips instead. Our contribution is not claiming to invent saliva-based cardiac sensing — it is applying a high-conductivity, biocompatible, antifouling-capable material platform to directly target the two biggest blockers (low sensitivity and short-lived sensor stability) that have kept this category from reaching continuous, multi-day wearable use.

# 8. Positioning Relative to the Closest Commercial Effort

RCE Technologies' Infrasensor is the most advanced non-blood cardiac biomarker device today, but it is a transdermal optical (infrared spectroscopy) device, not a chemical biosensor with a molecular recognition layer, and it is not yet FDA-cleared or commercially available. Our aptamer-based saliva platform offers a fundamentally different, more molecularly specific detection approach, and — unlike Infrasensor — is completely needle- and contact-free with skin.